SEABED SAMPLING

STATION NO 1

Appendix to form H575

Parameters

Date: 12 June 2016

Time: 06:34

Position: 57° 49.081' N, 7° 49.659' W (ETRS89, DD MM.MMM)

Water depth: 85.8 m

Sampler: Box Corer

Sample saved: No

Sampling comments

No sample collected after three attempts

Photo documentation

Description

SEABED SAMPLING

STATION NO 2

Appendix to form H575

Parameters

Date: 12 June 2016

Time: 05:57

Position: 57° 48.023' N, 7° 55.482' W (ETRS89, DD MM.MMM)

Water depth: 98.4 m

Sampler: Box Corer

Sample saved: Yes

Sampling comments

Sample collected on first attempt

Photo documentation

Description

Cobble and pebbles.

SEABED SAMPLING

STATION NO 3

Appendix to form H575

Parameters

Date: 12 June 2016

Time: 05:25

Position: 57° 44.716' N, 7° 55.776' W (ETRS89, DD MM.MMM)

Water depth: 132.3 m

Sampler: Box Corer

Sample saved: Yes

Sampling comments

Sample collected on first attempt

Photo documentation

Description

Fine sand, silt and broken shells.

SEABED SAMPLING

STATION NO 4

Appendix to form H575

Parameters

Date: 12 June 2016

Time: 04:48

Position: 57° 42.006' N, 8° 0.724' W (ETRS89, DD MM.MMM)

Water depth: 105.9 m

Sampler: Box Corer

Sample saved: Yes

Sampling comments

Sample collected on second attempt

Photo documentation

Description

Boulder and pebbles.

SEABED SAMPLING

STATION NO 5

Appendix to form H575

Parameters

Date: 12 June 2016

Time: 04:21

Position: 57° 42.2' N, 8° 4.379' W (ETRS89, DD MM.MMM)

Water depth: 123.2 m

Sampler: Box Corer

Sample saved: Yes

Sampling comments

Sample collected on first attempt

Photo documentation

Description

Medium sand and shells.

SEABED SAMPLING

STATION NO 6

Appendix to form H575

Parameters

Date: 12 June 2016

Time: 03:15

Position: 57° 39.696' N, 8° 0.929' W (ETRS89, DD MM.MMM)

Water depth: 101.2 m

Sampler: Box Corer

Sample saved: No

Sampling comments

No sample collected after three attempts

Photo documentation

Description

SEABED SAMPLING

STATION NO 7

Appendix to form H575

Parameters

Date: 12 June 2016

Time: 03:49

Position: 57° 39.58' N, 8° 8.348' W (ETRS89, DD MM.MMM)

Water depth: 131.7 m

Sampler: Box Corer

Sample saved: Yes

Sampling comments

Sample collected on first attempt

Photo documentation

Description

Medium and fine sand.

SEABED SAMPLING

STATION NO 8

Appendix to form H575

Parameters

Date: 12 June 2016

Time: 02:40

Position: 57° 37.836' N, 8° 3.6' W (ETRS89, DD MM.MMM)

Water depth: 105.3 m

Sampler: Box Corer

Sample saved: Yes

Sampling comments

Sample collected on first attempt

Photo documentation

Description

Cobble and pebbles.

SEABED SAMPLING

STATION NO 9

Appendix to form H575

Parameters

Date: 12 June 2016

Time: 02:17

Position: 57° 37.359' N, 8° 6.229' W (ETRS89, DD MM.MMM)

Water depth: 113.8 m

Sampler: Box Corer

Sample saved: Yes

Sampling comments

Sample collected on first attempt

Photo documentation

Description

Pebbles, coarse sand and broken shells.

SEABED SAMPLING

STATION NO 10

Appendix to form H575

Parameters

Date: 12 June 2016

Time: 01:11

Position: 57° 34.508' N, 8° 3.511' W (ETRS89, DD MM.MMM)

Water depth: 107.2 m

Sampler: Box Corer

Sample saved: Yes

Sampling comments

Sample collected on third attempt

Photo documentation

Description

Coarse sand and broken shells.

SEABED SAMPLING

STATION NO 11

Appendix to form H575

Parameters

Date: 12 June 2016

Time: 01:45

Position: 57° 35.285' N, 8° 10.662' W (ETRS89, DD MM.MMM)

Water depth: 137.7 m

Sampler: Box Corer

Sample saved: Yes

Sampling comments

Sample collected on first attempt

Photo documentation

Description

Fine and medium sand.

SEABED SAMPLING

STATION NO 12

Appendix to form H575

Parameters

Date: 12 June 2016

Time: 00:29

Position: 57° 33.804' N, 8° 6.672' W (ETRS89, DD MM.MMM)

Water depth: 117.1 m

Sampler: Box Corer

Sample saved: Yes

Sampling comments

Sample collected on first attempt

Photo documentation

Description

Cobble.

SEABED SAMPLING

STATION NO 13

Appendix to form H575

Parameters

Date: 11 June 2016

Time: 22:52

Position: 57° 29.752' N, 8° 3.541' W (ETRS89, DD MM.MMM)

Water depth: 100.8 m

Sampler: Box Corer

Sample saved: Yes

Sampling comments

Sample collected on third attempt

Photo documentation

Description

Granules, pebbles, broken shells and medium sand.

SEABED SAMPLING

STATION NO 14

Appendix to form H575

Parameters

Date: 11 June 2016

Time: 23:36

Position: 57° 31.169' N, 8° 7.01' W (ETRS89, DD MM.MMM)

Water depth: 129.1 m

Sampler: Box Corer

Sample saved: Yes

Sampling comments

Sample collected on third attempt

Photo documentation

Description

Coarse sand and broken shells.

SEABED SAMPLING

STATION NO 15

Appendix to form H575

Parameters

Date: 12 June 2016

Time: 00:00

Position: 57° 30.972' N, 8° 9.593' W (ETRS89, DD MM.MMM)

Water depth: 133.4 m

Sampler: Box Corer

Sample saved: Yes

Sampling comments

Sample collected on first attempt

Photo documentation

Description

Medium sand.

SEABED SAMPLING

STATION NO 16

Appendix to form H575

Parameters

Date: 11 June 2016

Time: 22:23

Position: 57° 27.848' N, 8° 4.903' W (ETRS89, DD MM.MMM)

Water depth: 103.4 m

Sampler: Box Corer

Sample saved: Yes

Sampling comments

Sample collected on second attempt

Photo documentation

Description

Granules, coarse sand, broken shells and pebbles.

SEABED SAMPLING

STATION NO 17

Appendix to form H575

Parameters

Date: 11 June 2016

Time: 21:53

Position: 57° 26.419' N, 8° 9.417' W (ETRS89, DD MM.MMM)

Water depth: 126.1 m

Sampler: Box Corer

Sample saved: Yes

Sampling comments

Sample collected on third attempt

Photo documentation

Description

Medium sand and broken shells.

SEABED SAMPLING

STATION NO 18

Appendix to form H575

Parameters

Date: 11 June 2016

Time: 21:16

Position: 57° 24.625' N, 8° 7.216' W (ETRS89, DD MM.MMM)

Water depth: 113.8 m

Sampler: Box Corer

Sample saved: Yes

Sampling comments

Sample collected on second attempt

Photo documentation

Description

Cobble and granules.

SEABED SAMPLING

STATION NO 19

Appendix to form H575

Parameters

Date: 11 June 2016

Time: 20:52

Position: 57° 23.245' N, 8° 3.491' W (ETRS89, DD MM.MMM)

Water depth: 103 m

Sampler: Box Corer

Sample saved: Yes

Sampling comments

Sample collected on second attempt

Photo documentation

Description

Pebbles.

SEABED SAMPLING

STATION NO 20

Appendix to form H575

Parameters

Date: 11 June 2016

Time: 19:50

Position: 57° 20.077' N, 8° 5.243' W (ETRS89, DD MM.MMM)

Water depth: 100.4 m

Sampler: Box Corer

Sample saved: Yes

Sampling comments

Sample collected on first attempt

Photo documentation

Description

Broken shells and coarse sand.

SEABED SAMPLING

STATION NO 21

Appendix to form H575

Parameters

Date: 11 June 2016

Time: 20:13

Position: 57° 21.377' N, 8° 10.218' W (ETRS89, DD MM.MMM)

Water depth: 113.9 m

Sampler: Box Corer

Sample saved: Yes

Sampling comments

Sample collected on first attempt

Photo documentation

Description

Cobbles.

SEABED SAMPLING

STATION NO 22

Appendix to form H575

Parameters

Date: 11 June 2016

Time: 19:24

Position: 57° 16.925' N, 8° 3.875' W (ETRS89, DD MM.MMM)

Water depth: 120.7 m

Sampler: Box Corer

Sample saved: Yes

Sampling comments

Sample collected on second attempt

Photo documentation

Description

Cobble and granules.

SEABED SAMPLING

STATION NO 23

Appendix to form H575

Parameters

Date: 11 June 2016

Time: 18:46

Position: 57° 17.777' N, 8° 7.023' W (ETRS89, DD MM.MMM)

Water depth: 137.9 m

Sampler: Box Corer

Sample saved: Yes

Sampling comments

Sample collected on first attempt

Photo documentation

Description

Fine sand.

SEABED SAMPLING

STATION NO 24

Appendix to form H575

Parameters

Date: 11 June 2016

Time: 18:22

Position: 57° 16.82' N, 8° 10.889' W (ETRS89, DD MM.MMM)

Water depth: 135 m

Sampler: Box Corer

Sample saved: Yes

Sampling comments

Sample collected on first attempt

Photo documentation

Description

Fine sand.

SEABED SAMPLING

STATION NO 25

Appendix to form H575

Parameters

Date: 11 June 2016

Time: 17:54

Position: 57° 13.899' N, 8° 10.051' W (ETRS89, DD MM.MMM)

Water depth: 114.9 m

Sampler: Box Corer

Sample saved: Yes

Sampling comments

Sample collected on first attempt

Photo documentation

Description

Medium sand and broken shells, one medium pebble.

SEABED SAMPLING

STATION NO 26

Appendix to form H575

Parameters

Date: 11 June 2016

Time: 17:32

Position: 57° 11.912' N, 8° 10.497' W (ETRS89, DD MM.MMM)

Water depth: 122.1 m

Sampler: Box Corer

Sample saved: Yes

Sampling comments

Sample collected on first attempt

Photo documentation

Description

Medium and fine sand.

SEABED SAMPLING

STATION NO 27

Appendix to form H575

Parameters

Date: 11 June 2016

Time: 11:44

Position: 57° 13.281' N, 8° 4.591' W (ETRS89, DD MM.MMM)

Water depth: 130.6 m

Sampler: Box Corer

Sample saved: Yes

Sampling comments

Sample collected on first attempt

Photo documentation

Description

Medium sand and broken shells.

SEABED SAMPLING

STATION NO 28

Appendix to form H575

Parameters

Date: 11 June 2016

Time: 12:25

Position: 57° 11.376' N, 8° 7.872' W (ETRS89, DD MM.MMM)

Water depth: 121.8 m

Sampler: Box Corer

Sample saved: Yes

Sampling comments

Sample collected on second attempt

Photo documentation

Description

Cobbles and pebbles.

SEABED SAMPLING

STATION NO 29

Appendix to form H575

Parameters

Date: 11 June 2016

Time: 13:22

Position: 57° 8.829' N, 8° 4.199' W (ETRS89, DD MM.MMM)

Water depth: 114.7 m

Sampler: Box Corer

Sample saved: Yes

Sampling comments

Sample collected on third attempt

Photo documentation

Description

Pebbles and granules.

SEABED SAMPLING

STATION NO 30

Appendix to form H575

Parameters

Date: 11 June 2016

Time: 17:00

Position: 57° 8.645' N, 8° 7.775' W (ETRS89, DD MM.MMM)

Water depth: 132.8 m

Sampler: Box Corer

Sample saved: Yes

Sampling comments

Sample collected on first attempt

Photo documentation

Description

Medium sand and broken shells.

SEABED SAMPLING

STATION NO 31

Appendix to form H575

Parameters

Date: 11 June 2016

Time: 16:35

Position: 57° 7.319' N, 8° 11.055' W (ETRS89, DD MM.MMM)

Water depth: 120.6 m

Sampler: Box Corer

Sample saved: Yes

Sampling comments

Sample collected on second attempt

Photo documentation

Description

Cobble and Pebbles.

SEABED SAMPLING

STATION NO 32

Appendix to form H575

Parameters

Date: 11 June 2016

Time: 13:55

Position: 57° 6.444' N, 8° 5.366' W (ETRS89, DD MM.MMM)

Water depth: 116.8 m

Sampler: Box Corer

Sample saved: Yes

Sampling comments

Sample collected on second attempt

Photo documentation

Description

Cobble.

SEABED SAMPLING

STATION NO 33

Appendix to form H575

Parameters

Date: 11 June 2016

Time: 15:58

Position: 57° 5.788' N, 8° 8.09' W (ETRS89, DD MM.MMM)

Water depth: 133.7 m

Sampler: Box Corer

Sample saved: Yes

Sampling comments

Sample collected on first attempt

Photo documentation

Description

Fine sand and silt.

SEABED SAMPLING

STATION NO 34

Appendix to form H575

Parameters

Date: 11 June 2016

Time: 15:33

Position: 57° 3.649' N, 8° 10.522' W (ETRS89, DD MM.MMM)

Water depth: 128.2 m

Sampler: Box Corer

Sample saved: Yes

Sampling comments

Sample collected on second attempt

Photo documentation

Description

Cobbles and pebbles.

SEABED SAMPLING

STATION NO 35

Appendix to form H575

Parameters

Date: 11 June 2016

Time: 15:05

Position: 57° 2.503' N, 8° 7.551' W (ETRS89, DD MM.MMM)

Water depth: 132.5 m

Sampler: Box Corer

Sample saved: Yes

Sampling comments

Sample collected on first attempt

Photo documentation

Description

Fine sand, silt and broken shells.

SEABED SAMPLING

STATION NO 36

Appendix to form H575

Parameters

Date: 11 June 2016

Time: 14:40

Position: 57° 0.93' N, 8° 4.802' W (ETRS89, DD MM.MMM)

Water depth: 130.9 m

Sampler: Box Corer

Sample saved: Yes

Sampling comments

Sample collected on first attempt

Photo documentation

Description

Fine sand, silt and broken shells.